Source Code :(LED Blinking)

void main() {

TRISB = 0x00;   // Configure all PORTB pins as output

PORTB = 0x00;   // Initially turn OFF all LEDs

while(1)

{

// Turn ON LED at RB0

PORTB.F0 = 1;

PORTB.F1 = 0;

PORTB.F2 = 0;

PORTB.F3 = 0;

Delay_ms(300);

// Turn ON LED at RB1

PORTB.F0 = 0;

PORTB.F1 = 1;

PORTB.F2 = 0;

PORTB.F3 = 0;

Delay_ms(300);

// Turn ON LED at RB2

PORTB.F0 = 0;

PORTB.F1 = 0;

PORTB.F2 = 1;

PORTB.F3 = 0;

Delay_ms(300);

// Turn ON LED at RB3

PORTB.F0 = 0;

PORTB.F1 = 0;

PORTB.F2 = 0;

PORTB.F3 = 1;

Delay_ms(300);

}

}

Circuit diagram:

Figure-1: LED flasher using PIC microcontroller.

Source Code :(Seven Segment Display)

char array[]={0x3f, 0x06, 0x5b, 0x4f, 0x66, 0x6d, 0x7d, 0x07, 0x7f, 0x6f};

int i;

void main() {

TRISB=0x00;

portb=0x00;

while(1)

{

for(i=0;i<10;i++)

{

portb=array[i];

delay_ms(1000);

}

}

}

Circuit Diagram:

Figure-2: count 0 to 9 in 7 segment display.

Source Code :

char array[]={0x3f, 0x06, 0x5b, 0x4f, 0x66, 0x6d, 0x7d, 0x07, 0x7f, 0x6f};

void main()

{

int i=0;

int j, Left_digit=0, Right_digit=0,count;

TRISB=0x00;

TRISC=0x00;

portb=0x00;

portc=0x00;

while(1)

{

Left_digit=i/10;

Right_digit=i%10;

for(j=0;j<50;j++)

{

portb=array[Left_digit];

portc.f0=0;

delay_ms(10);

portc.f0=1;

portc.f1=0;

portb=array[Right_digit];

delay_ms(10);

portc.f1=1;

}

if(i>99)

i=0;

else

i++;

}

}

Circuit Diagram:

Figure-3:7-segment multiplexer.

Source Code :

char arr[] = {0x40,0x79,0x24,0x30,0x19,0x12,0x02,0x78,0x00,0x10};

void main() {

int count=0,left,right;

TRISB = TRISC = 0x00;

TRISD = 0xff;

PORTB = PORTC = PORTD = 0x00;

while(1){

//increase button

if(PORTD.F0 == 1){

Delay_ms(30);          // debounce

if(PORTD.F0 == 1){

count++;

if(count == 100) count = 0;

while(PORTD.F0 == 1); // wait till release

}

}

//descrease button

if(PORTD.F1 == 1){

Delay_ms(30);          // debounce

if(PORTD.F1 == 1){

if(count == 0) count = 0;

else count--;

while(PORTD.F1 == 1); // wait till release

}

}

//reset button

if(PORTD.F2  == 1){

delay_ms(30);

if(PORTD.F2 == 1) {

count = 0;

}

}

left  = count/10;

right = count%10;

PORTC.F0 = 1;

PORTB = arr[left];    //segment show

Delay_ms(5);

PORTC.F0 = 0;

PORTC.F1 = 1;

PORTB = arr[right];

Delay_ms(5);

PORTC.F1 = 0;

}

}

Circuit Diagram:

Figure-4: interface double digit 7 segment LED display using push button.

Source Code :

void main() {

TRISB=0x00; //set port b as output

portb=0x00; //initialize as 0

while(1)

{

portb.f0=1;

delay_ms(1000);

portb.f0=0;

delay_ms(1000);

}

}

Circuit Diagram:

Figure-5: control a high voltage load using mechanical relay.

Source Code :

void main()

{

short duty = 0;

TRISD = 0xFF;

TRISB = 0x00;

TRISC.F2 = 0;

PORTB.F0 = 1;

PORTB.F1 = 0;

PWM1_Init(1000);

PWM1_Start();

PWM1_Set_Duty(0);

while(1)

{

// -- Speed UP (RD0) --

if(PORTD.F0 == 1)

{

Delay_ms(50);

if(PORTD.F0 == 1)

{

if(duty <= 245)

duty += 10;

PWM1_Set_Duty(duty);

while(PORTD.F0 == 1);  // wait release

}

}

// -- Speed DOWN (RD1) --

if(PORTD.F1 == 1)        // ? PORTD.F1 explicitly

{

Delay_ms(50);

if(PORTD.F1 == 1)

{

if(duty > 0)     // ? duty > 0 check

duty -= 10;

else

duty = 0;    // ? ???? negative ??? ??

PWM1_Set_Duty(duty);

while(PORTD.F1 == 1);  // wait release

}

}

}

}

Circuit Diagram:

Figure-6: DC motor speed using PIC microcontroller

Source Code :

int i;

void main() {

TRISB.F0 = 0;   // RB0 output

PORTB.F0 = 0;

while(1){

// 0 degree

for(i=0; i<50; i++){

PORTB.F0 = 1;

Delay_us(1000);   // 1 ms pulse

PORTB.F0 = 0;

Delay_us(19000);  // total 20 ms

}

Delay_ms(500);

// 90 degree

for(i=0; i<50; i++){

PORTB.F0 = 1;

Delay_us(1500);   // 1.5 ms pulse

PORTB.F0 = 0;

Delay_us(18500);

}

Delay_ms(500);

// 180 degree

for(i=0; i<50; i++){

PORTB.F0 = 1;

Delay_us(2000);   // 2 ms pulse

PORTB.F0 = 0;

Delay_us(18000);

}

Delay_ms(500);

}

}

Circuit Diagram:

Figure-7: Servo motor using PIC microcontroller

Source Code :

unsigned int adc_value;

char txt[7];

void main() {

ADCON1 = 0x80;        // AN0 analog

TRISA = 0xFF;         // PORTA input (AN0 input)

TRISC = 0x80;         // RC7 input, RC6 output

UART1_Init(9600);     // UART 9600 baud

Delay_ms(100);

while(1)

{

adc_value = ADC_Read(0);

IntToStr(adc_value, txt);  // Integer ? String

UART1_Write_Text("ADC Value = ");

UART1_Write_Text(txt);

UART1_Write(13);   // Carriage return

UART1_Write(10);   // New line

Delay_ms(500);

}

}

Circuit Diagram:(ADC)

Figure-8: Analog to Digital (ADC) conversion feature of PIC microcontroller.

Source Code :

// LCD module connections

sbit LCD_RS at RB0_bit;

sbit LCD_EN at RB1_bit;

sbit LCD_D4 at RB2_bit;

sbit LCD_D5 at RB3_bit;

sbit LCD_D6 at RB4_bit;

sbit LCD_D7 at RB5_bit;

sbit LCD_RS_Direction at TRISB0_bit;

sbit LCD_EN_Direction at TRISB1_bit;

sbit LCD_D4_Direction at TRISB2_bit;

sbit LCD_D5_Direction at TRISB3_bit;

sbit LCD_D6_Direction at TRISB4_bit;

sbit LCD_D7_Direction at TRISB5_bit;

// End LCD module connections

char txt1[] = "Information & Communication Engineering";

char txt2[] = "ICE(PUST)";

char i;                              // Loop variable

void Move_Delay() {                  // Function used for text moving

Delay_ms(300);                     // You can change the moving speed here

}

void main(){

ADCON1  = 0x06;                        // Configure AN pins as digital I/O

CMCON = 0x07;

Lcd_Init();

Lcd_Cmd(_LCD_CLEAR);

Lcd_Cmd(_LCD_CURSOR_OFF);

Lcd_Out(1,1,txt1);

Lcd_Out(2,5,txt2);

Delay_ms(1000);

while(1) {                         // Endless loop

for(i=0; i<24; i++) {

Lcd_Cmd(_LCD_SHIFT_LEFT);

Move_Delay();

}

/* for(i=0; i<8; i++) {             // Move text to the right 7 times

Lcd_Cmd(_LCD_SHIFT_RIGHT);

Move_Delay();

}   */

}

}

Circuit Diagram:(LCD)

Figure-9: Interface and Display Data on LCD using PIC Microcontroller.

Source Code :

// LCD module connections

sbit LCD_RS at RB0_bit;

sbit LCD_EN at RB1_bit;

sbit LCD_D4 at RB2_bit;

sbit LCD_D5 at RB3_bit;

sbit LCD_D6 at RB4_bit;

sbit LCD_D7 at RB5_bit;

sbit LCD_RS_Direction at TRISB0_bit;

sbit LCD_EN_Direction at TRISB1_bit;

sbit LCD_D4_Direction at TRISB2_bit;

sbit LCD_D5_Direction at TRISB3_bit;

sbit LCD_D6_Direction at TRISB4_bit;

sbit LCD_D7_Direction at TRISB5_bit;

// End LCD module connections

char display[16]="";

void main()

{

unsigned int result;

float volt,temp;

trisb=0x00;

trisa=0xff;

adcon1=0x80;

lcd_init();

lcd_cmd(_lcd_clear);

lcd_cmd(_LCD_CURSOR_OFF);

while(1)

{

result=adc_read(0);

volt=result*4.88;

temp=volt/10;

lcd_out(1,1,"Temp = ");

floattostr(temp,display);

lcd_out_cp(display);

lcd_chr(1,16,223); //print at pos(row=1,col=13) "°" =223 =0xdf

lcd_out_cp(" C"); //celcius

}

}

Circuit Diagram:(Temperature sensor)

Figure-10: sensor input from LM35 temperature sensor and display corresponding sensor value through LCD display.

Source Code :

void main()

{

TRISD = 0x00;

PORTD = 0x00;

do

{

PORTD = 0b00000011;

Delay_ms(500);

PORTD = 0b00000110;

Delay_ms(500);

PORTD = 0b00001100;

Delay_ms(500);

PORTD = 0b00001001;

Delay_ms(500);

} while(1);

}

Circuit Diagram:(Stepper Motor)

Figure-11: Stepper motor using PIC microcontroller.

Source Code :

unsigned char count;

unsigned char seg[] = {0x3F,0x06,0x5B,0x4F,0x66,0x6D,0x7D,0x07,0x7F,0x6F};

void main(){

ADCON1 = 0x06;

TRISB = 0x00;

count = EEPROM_Read(0x00);

if(count > 9) count = 0;

while(1){

PORTB = seg[count];

Delay_ms(1000);

count++;

if(count > 9) count = 0;

EEPROM_Write(0x00, count);

Delay_ms(20);

}

}

Circuit Diagram:(EEPROM)

Figure-12: Interface EEPROM for Data Storage and Retrieval using PIC Microcontroller

Source Code :

void MSDelay(unsigned char Time){

unsigned char y,z;

for(y=0x00; y<Time; y++)

for(z=0x00; z<0x14; z++);}

void main(){

TRISC = 0x00;

TRISD = 0x00;

while(0x01)  {

PORTD = 0x80;

PORTC = 0xFF;

MSDelay(0x0A);

PORTD = 0x40;

PORTC = 0xFF;

MSDelay(0x0A);

PORTD = 0x20;

PORTC = 0x3C;

MSDelay(0x0A);

PORTD = 0x10;

PORTC = 0x66;

MSDelay(0x0A);

PORTD = 0x08;

PORTC = 0xC3;

MSDelay(0x0A);

PORTD = 0x04;

PORTC = 0x81;

MSDelay(0x0A);

PORTD = 0x02;

PORTC = 0x00;

MSDelay(0x0A);

PORTD = 0x01;

PORTC = 0x00;

MSDelay(0x0A);   }}

Circuit Diagram:(Dot Matrix Display)

Figure-13: display character through dot matrix display.